Yes — exactly, that's a very important point and needs to be integrated into the existing logic:

> Alerts should never be based solely on a single sensor value. They must always consider the machine/production state first.

I'll add this as an additional component without removing anything from the existing logic.

--

Additional Extension – Machine State as a Mandatory Prerequisite for Alerts

Objective

The alert system must not react blindly to pressure, temperature, or other sensors.

Before an alert is triggered, the system must always first check:

what machine state the system is in

whether this state is even relevant for an alert

which sensor ranges are expected/normal in this state

This prevents false alarms.

--

Basic Principle

A sensor value is not inherently good or bad.

Its significance depends on the current machine state.

Examples:

Low melt pressure, but machine is OFF / IDLE / COOLING

→ no pressure alert

Low melt pressure, but machine is PRODUCTION

→ warning or critical

High melt pressure, but machine is HEATING UP / Starting up

→ possibly normal / just monitoring

High melt pressure, machine is PRODUCTION

→ warning / critical depending on the material profile

---

Machine status as the first check criterion

The alert logic must always use this sequence:

1. Check machine states

2. Check material/batch/silo context

3. Check sensor value against configured range

4. Derive alert / email / dashboard color from this

---

Machine states

The system already works with states such as:

OFF

READY

HEATING UP

PRODUCTION

COOLING UP

These states must now be directly linked to the alert logic.

---

Alert Relevance per Machine State

1. OFF

When the machine is OFF:

No production alert

No printing alert

No temperature warning due to lack of production

No quality warning

Optionally, only technical alerts are triggered if something is completely implausible.

... Expected Behavior

Low or zero pressure = normal

Speed ​​zero = normal

Temperature dropping = normal

---

2. READY

When the machine is in READY:

No production evaluation

No normal quality alerts

Low pressure can be normal

Temperatures can be in the standby range

Alerts only if explicitly configured:

Unusually high pressure in standby mode

Extreme temperature deviation

---

3. HEATING UP

When the machine is in HEATING UP:

Temperature profiles may still be outside the production range

Pressure is often low or unstable

No normal pressure alerts as in production mode

No quality evaluation active

Optional:

If heating takes too long

If target temperature ranges are not reached

If unusual conditions occur

---

4. PRODUCTION

Full material and process evaluation should only be active in PRODUCTION mode.

Then the following apply:

Melt pressure limits

Temperature zone limits

Screw speed limits

Dosing

Motor load

Stability

Baseline comparison

AI marker / Process classification

Quality and wear risk

Important sentence

All normal alerts should primarily only be active in PRODUCTION mode.

---

5. COOLING

When the machine is COOLING:

Pressure drops = normal

Temperatures drop = normal

Speed ​​low / zero = normal

Do not trigger any production alerts.

Optional only:

if the condition lasts an unusually long time

if the temperature does not drop

if pressure remains high despite cooling

---

Configurability of the alert logic according to machine state

This logic should also be configurable.

This means:

For each sensor or parameter, it should be possible to define in which machine states it is relevant for triggering an alert.

---

Example – Melting Pressure

For melting pressure, the following should be configurable in the material/parameter profile:

Alert active in states:

PRODUCTION = yes

HEATING UP = optional

READY = no

OFF = no

COOLING DOWN = no

This prevents low pressure from being incorrectly interpreted as a problem when the machine is stationary.

---

Example – Temperature Zones

For extruder zones 1–5, a further differentiation can be made later:

Alert active in states:

PRODUCTION = yes

HEATING UP = partially yes

READY = optional

OFF = no

COOLING DOWN = no

Because temperature is already relevant during the heating phase, but pressure often isn't.

---

Example – Screw Speed

For screw speed:

PRODUCTION = yes

READY = no

OFF = no

COOLING = no

HEATING UP = optional

---

Extension in the configuration mask

Under "Configure material" or in the parameter profile, an additional section should be added:

"Alert behavior based on machine status"

There, you can configure the following for each sensor:

Example UI text in German

Alerts in OFF state active

Alerts in READY state active

Alerts in HEATING up state active

Alerts in PRODUCTION state active

Alerts in COOLING state active

Or more concisely:

Alert-relevant states

Multiple selection:

OFF

READY

HEATING UP

PRODUCTION

COOLING DOWN

---

Connection to material, baseline, and alert

This complete logic now results:

1. Check machine state

Is the sensor even relevant in this state?

2. Load Production Context

Material

Batch

Silo / Big Bag

Profile / Product

Quality

Color

3. Load Material Profile / Override

The configured target values, normal ranges, warning ranges, and critical ranges

4. Baseline Comparison

Comparison against:

Material profile as a technical reference

Learned baseline as a data-driven refinement

5. Alert Decision

Only if all conditions are met:

Condition appropriate

Sensor out of range

Poor stability if present

Critical AI marker if present

Then:

Dashboard color

Warning

Critical warning

Email

---

Technical Alert Decision Logic

The alert engine should operate as follows:

Step 1

Check current machine status

Step 2

Check if the sensor is alert-relevant in this state

If no:

None Warning

No email alert

Display/Monitoring only

Step 3

If yes:

Load active material profile

Check batch/silo/running context

Load value ranges

Compare current measured value

Step 4

Calculate status:

Normal range

Warning range

Critical range

Step 5

Trigger alert if enabled in the profile

---

Why this is important

Without this state logic, false alerts occur, such as:

Low pressure during standstill

Falling temperature during cooling

Zero rotation speed in standby mode

Unstable values ​​during changeover

These are not errors, but normal states.

The system must therefore always distinguish between:

Correct state

and

Process problem

---

Additional AI relevance

The machine state is also extremely important for AI/process classification.

The AI ​​must not learn all data blindly

It should only use specific phases for specific models:

For quality and production analysis:

PRODUCTION only

For start-up optimization:

HEATING UP / Start-up phase only

For shutdown analysis:

COOLING DOWN only

For wear analysis:

Primarily compare stable PRODUCTION phases

---

Naming recommendation

You asked if "under Material" is really the best structure.

My honest recommendation:

In the long run, a better naming convention is:

"Configure production profile"

or

"Configure process profile"

Why?

Because it's not just about material, but about:

Material

Batch

Silo / Big Bag

Product / Profile

Quality context

Process parameters

Alert rules

Baseline reference

Technically, this is actually a complete production profile, not just a material entry.

But for the first step,

you can leave it under "Material" because it's clearer for the user.

You can rename it properly later.

--

Additional rule for the developer – English

Additional rule: alerts must always be state-aware.

Before evaluating any sensor against thresholds, the system must first check the current machine state.

A sensor must only trigger alerts if alerts are enabled for that sensor in the current machine state.

Example:

Low melt pressure during OFF / READY / COOLING must not trigger a production alert.

Low melt pressure during PRODUCTION can trigger a warning or critical alert.

Temperature deviation during HEATING may be handled differently than during PRODUCTION.

Please extend the configuration model so that for each relevant sensor, we can define in which machine states alerts are active.

This state-aware logic must be applied before:

threshold comparison

baseline comparison

alert generation

email notifications

This must work together with:

material profile

charge / silo / production context

baseline logic

future AI process classification

---

If you'd like, I can now write you the complete developer specification in one go—that is, everything so far plus this new state-aware alert logic, in a single, clean version in English, with all UI terms translated into German.