Exactly. That's the key point:

For the future, it's easy because the production manager enters it directly.

For the past, you have to reconstruct labels retrospectively.

This isn't done by blindly adding everything manually, but rather like this:

Goal

You want to break down historical time series into production runs and then have the production manager confirm only the missing contextual information for each run:

Material

Batch

Silo / Big Bag

Profile / Product

Quality

Special Features

Maintenance / Cleaning / Changeover

---

The Right Approach

1. First, automatically segment historical data into runs

You mustn't show the production manager raw data, but rather pre-defined production phases.

The system should automatically create blocks from the historical data, like this:

Run A: March 12, 8:10 AM – March 12, 2:25 PM

Run B: March 12 15:05 – 13.03. 02:20

Run C: 14.03. 07:40 – 14.03. 18:10

You can identify these runs using rules such as:

Screw speed > X

Melt pressure > Y

Temperature > Z

continuous over a minimum duration

Therefore:

First, automatically determine the run limits, then label them.

---

2. Automatically generate a summary for each run

For each detected run, you automatically calculate:

Start time

End time

Duration

Average pressure

Pressure min/max

Average screw speed

Average motor load

Average temperature zone 1–5

Temperature spread

Number of dips/anomalies

And very importantly:

Screenshot/mini-chart of the run

Estimated pressure class:

low pressure

medium pressure

high pressure

Then you don't show the production manager 100,000 measurements, but something like this:

Run 2026-03-12-01

08:10–14:25

Avg. pressure 342 bar

Avg. RPM 10.2

Avg. temp 175°C

Class: Low Pressure

Question:

What material?

What batch?

Which profile?

Special features?

--

3. Provide the production manager with a review for

You need a simple review interface or table.

Per detected run:

Time period

Mini-chart

Calculated key figures

Assumed class

Input fields:

Material

Batch

Silo / Big Bag

Profile / Product

Quality

Color

Recycled yes/no

Maintenance / Cleaning / Filter change

Remarks

The production manager then only needs to confirm or add to the information.

This is much more realistic.

---

How to Link Historical Data with New Labels

You need a new structure:

Table 1: production_runs

One data record per detected run:

run_id

machine_id

start_time

end_time

duration_min

avg_pressure

avg_rpm

avg_temp

pressure_class_guess

status

Table 2: production_run_labels

What the production manager adds:

run_id

material

charge

silo_bigbag

profile_product

quality

color

recycled_material

maintenance_event

operator_note

confirmed_by

confirmed_at

Then all historical sensor data can be assigned to this run by time range or run_id.

--

How the assignment works technically

There are two ways.

Method A: Time Range

All measurements with a timestamp between:

start_time

end_time

belong to this production run.

This is the simplest method and sufficient for initial use.

Method B: Write run_id directly into measurements

More difficult to add for historical data retrospectively, but excellent for future use.

For past data:

Link via time range

For future data:

Actively include run_id

---

How to automatically filter runs

You need simple rules.

Example: Production run is active if:

pressure_bar > 300

screw_speed_rpm > 5

temp_zone_avg > 150

If these conditions are met for more than 10 minutes: → Production run begins

If they are not met for more than 10 minutes: → Production run ends

This is how you automatically generate blocks.

---

How to make the production manager's job easier

Not 200 runs at once.

Instead:

Prioritize

First, show only these historical runs:

1. Longest runs

2. Runs with high pressure

3. Runs with a sharp pressure increase

4. Unstable runs

5. Runs just before maintenance/problems

This way, you get the most important labels first.

--

How to semi-automatically guess product labels

Before you ask, give the system a guess.

For example:

Heuristics

Average pressure 300–350 → likely low-pressure material

Average pressure 340–380 → likely medium-pressure material

Average pressure 370–400 → likely high-pressure material

Temperature 170–175 → Material group A

Temperature 185–195 → Material group B

Then you show:

Assumption: Medium pressure / Material group B

The production manager only needs to make corrections.

This saves a significant amount of time.

---

Other sources for reconstructing the past

In addition to sensor data, you should compare these sources:

Production sheet / shift log

ERP / production order

Excel spreadsheets

Maintenance log

Emails / WhatsApp / shift notes

Handwritten production parameter sheets

Delivery note / Big bag / silo documentation

Often, you'll at least get the following information:

Date

Product

Material

Batch

Then you match this with the time range.

--

Practical workflow for you

Phase 1

Automatically detect historical production runs

Phase 2

Generate key performance indicators (KPIs) and a mini-chart for each run

Phase 3

Production manager receives a review list

Phase 4

They add/confirm material, batch, profile, etc.

Phase 5

These labels are attached to the historical time series

Phase 6

You'll train on:

Baselines

Process classes

Drift

Quality risk

Wear and tear

---

What you should build first

Don't create the perfect template for everything right away, but start with:

1. Run detection

Automatic

2. Run summary

Automatic

3. Review template

Simple

Fields:

Material

Batch

Silo / Big Bag

Profile / Product

Quality

Remarks

4. Saving

Using run_id

This gives you the foundation.

---

Important for AI

An AI model doesn't need perfect single-point labels right away.

Often, it's enough to know the following for each run:

what ran

which batch

what quality

whether good/bad/noticeable

Run-level labels are much more realistic to begin with than labeling every second.

--

Conclusion

You don't have to completely recreate the historical data manually.

You should:

first, automatically detect production runs

condense these

show the production manager only the condensed runs

have the material/batch/product confirmed for each run

then append these labels to the history by time range

This is the right and feasible approach.

If you'd like, I can now write you a concrete specification for the historical labeling interface so your developer knows exactly what this review interface should look like.